American Dream Vacations

7310 E. Ben White Blvd., TX  78741   Phone: 512-385-3680

E-Mail:  kurt@americandreamvacations.net   Website:  www.americandreamvacations.net

We find that understanding the electrical components of the RV operating systems is one of the most confusing yet important parts of the RV experience. Partially drained batteries can cause a wide-range of issues.

Generally, there are three ways to ensure your RV is charging the batteries. If the truck engine is on it will send a charge to the house battery bank. Towable will have take a charge from the tow vehicle if the 7-way connector’ charge line has been hooked up.

Running the generator while in travel allows full RV functionality and battery charge. Once at your destination plug into electrical shore power right away. The RV is not designed to operate solely on the 12V battery without taking a charge—dry camp at your own risk.

Below is an amperage usage chart showing what different appliances will draw from the RV power systems. Remember good power management greatly reduces the chance of an electrical failure in an RV.

One other thing to keep in mind is many RV appliances require more amps to start the appliance than they do to run the appliance. A roof air conditioner can draw 16 amps to start, but may only use 13 amps once it is running.

*Statistics below provided by KOA.com/RV Services

| 120 Volt AC Amp Ratings |  |  |
| --- | --- | --- |
| Appliance or Electronic Equipment | Estimated Amps |  |
| Air Conditioner (X number of A/C) | 12-16 Amps |  |
| Blender | 5-6 Amps |  |
| Coffee Maker | 5-8 Amps |  |
| Compact Disc Player | 1 Amp |  |
| Computer (Laptop) | 2-3 Amps |  |
| Converter | 1-8 Amps |  |
| Crock Pot | 1-2 Amps |  |
| Curling Iron | <1 Amp |  |
| Drill | 2-6 Amps |  |
| Electric Blanket | 0.5-1.5 Amps |  |
| Electric Fan | 1 Amp |  |
| Electric Water Heater | 9-13 Amps |  |
| Electric Skillet | 6-12 Amps |  |
| Hair Dryer | 5-12 Amps |  |
| Iron | 5-10 Amps |  |
| Light (60 watt % 120V) | <1 Amp |  |
| Microwave | 8-13 Amps |  |
| Microwave (Convection Oven) | 13 Amps |  |
| Refrigerator in AC mode | 5-8 Amps |  |
| Space Heater | 8-13 Amps |  |
| Television | 1.5-4 Amps |  |
| Toaster | 7-10 Amps |  |
| Vacuum (handheld) | 2-6 Amps |  |
| VCR | 1-2 Amps |  |
| Washer/Dryer | 14-16Amps |  |
| 12 Volt DC Amp Ratings |  |  |
| Appliance or Accessory | Estimated Amps |  |
| Aisle Light | 1 Amp |  |
| CO Detector | 1 Amp |  |
| Fluorescent Light | 1-2 Amps |  |
| Furnace | 10-12 Amps |  |
| LP Gas Leak Detector | 1 Amp |  |
| Overhead lights (Per Bulb) | 1 Amp |  |
| Porch Light | 1 Amp |  |
| Power Roof Vent | 1.5 Amps |  |
| Radio/Stereo | 4 Amps |  |
| Range Hood (Fan & Light) | 2-3 Amps |  |
| Refrigerator (LP Gas Mode) | 1.5- 2 Amps |  |
| Security System | 1 Amp |  |
| Television (12 volt) | 4-5 Amps |  |
| TV Antenna Booster | <1 Amp |  |
| TV Antenna Booster 12 Volt outlet | Up to 8 Amps |  |
| Variable Speed Ceiling / Vent Fan | 4 Amps |  |
| VCR Recorder / Player | 2 Amps |  |
| Water Pump | 4 Amp |  |

Hopefully this clears up any confusion on the difference between using amps supplied by the campground electrical source (AC amps), and using amps supplied by the converter (DC Amps).

Happy Camping!